User’s manual

[image: image1.png]


M²M Tec >>M²M-box
Thank you for using the M²M-box. Please read this user’s manual carefully before use.
                  Note: Never use the M²M-box within the following areas:

            [image: image4.wmf]
 
Areas where a wireless transceiver is forbidden to use or 

needs to be shut down. 

Gas stations.    
Storage areas with flammable and explosive stuff.
Other environments in which danger maybe caused  
                    by wireless communication.
1. Functions & Features
M²M-box is an innovative product build for the elevator/lift industry 

M²M-box has the following functions：
1). Support GSM900/1800 or 850/1900MHz.

2). Provide the reverse polarity signal and can connect with the charge counter for charging
3). Can set up call-out/call-in restriction
4). Caller ID 
5). Hands free, redial, sound volume adjustment while calling in/out.
7). Can connect with PBX

8). With back-up Li battery, support standby time over 48hours.
2. Configurations

Standard M²M-box includes the following accessories:

	Accessories
	Quantity

	Single-channel M²M-box mainframe
	1

	Antenna
	1

	SIM cardboard
	1

	Telephone cable
	2

	Adapter
	1

	User’s manual
	1

	Bolt
	3


3. M²M-box Installation
3.1 SIM card installation:

Caution!  Please fix the SIM card before connecting to the power supply!

1). Upend the mainframe so you can see the SIM card pedestal at the bottom.
2). Push SIM card nog gently along with arrowhead to put it up.

3). Insert SIM card into the nog. 

4). Press down the nog of SIM card and push it by the backward direction of arrowhead until the SIM nog is fixed on the card pedestal.

5). Use the SIM cardboard to cover pedestal and screw it down.

3.2 Assembly of the terminal:

[image: image2.jpg]O —

Antenna off/on Power Phone1 Phone2




       

                               External ports
1). Make sure the SIM card installed correctly.

2). Screw down the connector of the antenna to the bolt stick on the panel marked with “Antenna” on the back of the device.

3). Plug one connector of standard phone cable to the phone, and plug the other end to the port on the panel marked with “Phone” on the back of the device.

4). Plug one connector of standard phone cable to the phone, and plug the other end to the port on the panel marked with “Phone” on the back of the device. 

  The model support connect with 2pcs normal telephone as extension at same time

5). Make sure the switch of device is off, then plug the input connector of the adaptor to the AC power socket, and the output connector to the “Power” port on the back of the device.

4. Operation Instructions

Turn on the power switch at the back of the M²M-box after connecting the device correctly

4.1 Explanation of indicators’ state：

 [image: image3.jpg]Signal intensity

=2
1]
—+
g
o
=
kY




                          Indicators

1) The route indicator indicates which network is working - GSM or PSTN. (optional)

  This light is on while the device is operated under GSM network, vice versa.

2) The network indicator indicates whether the device get connected with GSM network or not.

   This light is on means the device gets connected with GSM network, vice versa.
3) The signal intensity indicators display signal intensity. 

When the signal is stronger, more indictor lights will be on, vice versa.

4.2 Dial the telephone:
When the network and signal intensity indicators are on, pick up the handset and dial after hearing the dialing tone. .

4.3 Answer a telephone call:
When the telephone rings on, answer the phone by picking up the handset.

4.5. Built-in Li-battery:

It will automatically charge the Li-battery when the device is connected with AC adaptor and turned on.

5. Setting Functions
This M²M-box can set up and adjust the following functions：
1). Can adjust the multi-level sound volume.

2). Support to change the hot line number.

3). Support to change the message receiver number.

Caution! The system setup must be operated by the professional person. 

Technical Parameters

	Items
	Parameters

	Working voltage
	AC100~240V; DC12V 

(Adaptor is enclosed according to local standard voltage. Before use, please make sure the adaptor voltage matches with local voltage.)

	Environment temperature
	0~45℃

	Humidity
	40%~95%

	Atmospheric pressure
	85~110Kpa

	Environment noise
	<60dB

	Eradiation power
	2W

	Sensitivity
	-104dBm

	Antenna gain
	>5dB


Troubleshooting
	No.
	Problem
	Possible Causes
	Possible Remedy

	1
	Not connected to GSM network.
	SIM card not inserted correctly.

SIM card is dirty.

Antenna is not well connected.
	Install the SIM card again.

Clean the SIM card with a dry cloth.

Check the antenna connection.

Adjust the antenna position.

	2
	No dialing tone is heard after off-hook.
	1. The phone cable is not correctly connected.
	1. Check the cable connection.

	3
	No caller ID.
	The service is not enabled.

The phone does not support caller ID.
	Check the service is enabled on the GSM network.

Change for a phone supporting caller ID.


Notes

Working environment: Choose areas with good signal 
Requirement for the power supply: The reliable and stable power supplies will guarantee the normal working of  M²M-box.
Antenna: Keep the antenna far away from the telephone to reduce the communication disturbance. 
   


       Start Communicating  











9
-- 1- -


